C-130 Deployment to a Forward Landing Strip (FLS)

While the C-130 landing weight for improved runways during peacetime is 155,000 pounds; the peacetime landing weight on a FLS is 130,000 pounds.  This becomes critical for  the transport of C-130 capable equipment for both training and during deployment.  The last time the 130,000 pounds FLS limitation was waived (allowing 155,000 pounds at a FLS) was during DESERT STORM.  All deployments since then have been accomplished using peacetime limits.

The 130,000 pound limitation results in a payload weight of about 32,000 pounds being delivered to an FLS when refueling is not available at the FLS, considering  the aircraft must fly approximately 1 hour to an airbase for refueling.  This can eliminate consideration of C-130 transport of vehicles weighing significantly in excess of 32,000-pounds for missions such as deployment to Bosnia and Kosovo.  It also limits the ability to train during peacetime.

The airfield requirement for the 155,000-pound capability of the C-130 is use of a paved landing strip of at least 3,000-foot length and 60 foot width for tactical assault operations.  Non-tactical assault operations require a 5,000-foot length and 80 foot width runway.

The FLS limitation has not been a significant C-130 restriction in the past as there have been few combat systems with a weight greater than 32,000 pounds that were C-130 capable.  The current emphasis on light combat vehicles that are C-130 capable makes the 130,000-pound landing limitation a significant  operating and training restriction.