### The LAV III is NOT ‘transportable’ on C-130 Aircraft –

Why?

Because: The Army’s Military Traffic Management Command (MTMC) says so:  (Ref. B., pp 7 - 8)

- At 38,000 lb, it is too heavy.

- At 105 in. high, it is too tall.

- At 105 in. wide, it is too wide at the permanent floor ‘463L’ pallet rails.

MTMC Data Details:

|  | Max Wt (lb) (A.4. *) | Max Ht (in.) | Max Width (in.) f/ Roll-on/Roll-off |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | At Body/ Chassis | At floor rails |  |
|  |  |  |  | ‘Wheels’ | ‘Tracks’ |
| # C-130 a/c | 32,000 (A.4 *) | 102 (A.3) | 105 (A.3) | 102 (A.3) | 100 (A.3) |
| LAV III | 38,000 | 105 ** | 105 | 105 | ------------- |
| M113A3 (B.2) | 23,900 | 99 | 100 | ------------- | 100 (A.3) |

* See last Para on page 4, Ref. A.

** Height may be reducible.

References:

A.  Welcome To Deployability Engineering (Fixed Wing Air).  Issued by Transportation Engineering Agency,

Military Traffic Management Command.  See URL:  http://www.tea.army.mil/dpe/Aircraft.htm
A.3. Page 3 of Ref. A.

#### A.4. Page 4 of Ref. A.  Effective ‘Allowable Cabin Load’ =

Maximum payload C-130 can fly into forward landing strips w/o refueling facilities.

B. Welcome To Deployability Engineering (Air Transportability).  Issued by Transportation Engineering Agency, 
Military Traffic Management Command.  See URL:  http://www.tea.army.mil/dpe/Aircraft%20loads.htm

B.2. Page 2 of Ref. B.

## Revised:  January 18, 2001;  gg-010118B.doc